#include <16f887.h>

#fuses INTRC_IO, NOWDT, NOPUT, NOBROWNOUT, NOPROTECT

#device adc=10

#use delay(clock=4M)

#use rs232(baud=9600, xmit=pin_c6, rcv=pin_c7, bits=8,parity=N, ERRORS)

#use i2c(Master,Fast=100000, sda=PIN_c2, scl=PIN_c1,force_sw)

#include "caliberi2c.c"

//INT H;

VOID MAIN()

{

FLOAT b1,b2,b3,t,v,c,s1,s2,s3,b;

lcd_init();

lcd_backlight_led(ON); //Backligth ON

lcd_init();

lcd_init();

setup_adc(ADC_CLOCK_INTERNAL ); // initialize ADC with a sampling rate of Crystal/4 MHz

setup_adc_ports(sAN0,sAN1);

setup_adc_ports(sAN2,sAN3);

setup_adc_ports(sAN4,sAN5);

setup_adc_ports(sAN6,sAN7);

delay_ms(500);

lcd_init();

printf(lcd_putc,"\fWELCOME ");

delay_ms(1000);

WHILE(1)

{

set_adc_channel(6);

delay_ms(10);

b1=read_adc();

delay_ms(10);

b1=(b1/1023)*12;

delay_ms(10);

//b1=b1-1;

set_adc_channel(1);

delay_ms(10);

b2=read_adc();

delay_ms(10);

b2=(b2/1023)*6;

delay_ms(10);

//s=v*7.2;

set_adc_channel(2);

delay_ms(10);

b3=read_adc();

delay_ms(10);

b3=(b3/1023)*11.5;

delay_ms(10);

b=b1+b2+b3;

s1=b1*27;

s2=b2*27;

s3=b3*27;

set_adc_channel(3);

delay_ms(10);

t=read_adc();

delay_ms(10);

t=(t/1023)*300;

delay_ms(10);

set_adc_channel(4);

delay_ms(10);

v=read_adc();

delay_ms(10);

v=(v/1023)*15;

delay_ms(10);

set_adc_channel(5);

delay_ms(10);

c=read_adc();

delay_ms(10);

c=(c/1023)*5;

delay_ms(10);

if((t>45))

{

printf(lcd_putc,"\ftemp high\nchgr....off");

delay_ms(1500);

printf(lcd_putc,"\ftemp high\ncooling fan on");

delay_ms(1500);

}

else

{

}

if((b>11))

{

printf(lcd_putc,"\fover\nchgr....off");

delay_ms(1500);

}

else

{

}

if((b<8)&&(t<44))

{

output_high(pin_d0);

delay_ms(100);

}

else

{

output_low(pin_d0);

}

if((t>45)||(b>11))

{

output_low(pin_d0);

delay_ms(100);

}

else

{

//output_low(pin_d0);

}

if((t>45))

{

output_high(pin_d1);

delay_ms(100);

}

else

{

output_low(pin_d1);

}

if((b1<2))

{

printf(lcd_putc,"\fcell 1 low \n bms active ");

delay_ms(1500);

}

else

{

}

if((b2<2))

{

printf(lcd_putc,"\fcell 2 low \n bms active ");

delay_ms(1500);

}

else

{

}

if((b3<2))

{

printf(lcd_putc,"\fcell 3 low \n bms active ");

delay_ms(1500);

}

else

{

}

printf("%f,%f,%f,%f,%f,%f,\n",b1,b2,b3,t,v,c);

printf(lcd_putc "\fB1:%f B2:%f\nB3:%f",b1,b2,b3);

delay_ms(1500);

printf(lcd_putc "\fT:%f CV:%f\nCC:%f",t,v,c);

delay_ms(1500);

printf(lcd_putc "\fSOC1:%f\nSOC2:%f"s1,s2);

delay_ms(1500);

printf(lcd_putc "\fSOC3:%f\nBV:%f"s3,b);

delay_ms(1500);

}

}