BUZZER:

GENERAL DESCRIPTION:

A buzzer or beeper is an audio signaling device, which may be mechanical, electromechanical, or piezoelectric. Typical uses of buzzers and beepers include alarm devices, timers and confirmation of user input such as a mouse click or keystroke.

Buzzer is an integrated structure of electronic transducers, DC power supply, widely used in computers, printers, copiers, alarms, electronic toys, automotive electronic equipment, telephones, timers and other electronic products for sound devices. Active buzzer 5V Rated power can be directly connected to a continuous sound, this section dedicated sensor expansion module and the board in combination, can complete a simple circuit design, to "plug and play."

Now, micro computers are widely used for microwave ovens, air conditioners, cars, toys, timers, and other alarm equipment. Externally driven piezoelectric sounders are used in digital watches, electronic calculators, telephones and other equipment. They are driven by a signal (ex, 2048Hz or 4096Hz) from an LSI and provide melodious sound.

FEATURES

1. Low power consumption.

2. No contacts therefore, no noise and highly reliable.

APPLICATIONS

• Telephone ringers.

• Various office equipment such as PPCs, printers and keyboards.

• Various home appliances such as microwave ovens.

• Confirmation sound of various audio equipment.

SPECIFICATIONS:

• On-board passive buzzer

• On-board 8550 triode drive

• Can control with single-chip microcontroller IO directly

• Working voltage: 5V