SOFTWARE REQUIREMENT:

EMBEDDED C :

Embedded C is one of the most popular and most commonly used Programming Languages in the development of Embedded Systems. So, in this article, we will see some of the Basics of Embedded C Program and the Programming Structure of Embedded C.

Embedded C is perhaps the most popular languages among Embedded Programmers for programming Embedded Systems. There are many popular programming languages like Assembly, BASIC, C++ etc. that are often used for developing Embedded Systems but Embedded C remains popular due to its efficiency, less development time and portability.

Before digging in to the basics of Embedded C Program, we will first take a look at what an Embedded System is and the importance of Programming Language in Embedded Systems.

EMBEDDED SYSTEM:

An Embedded System can be best described as a system which has both the hardware and software and is designed to do a specific task. A good example for an Embedded System, which many households have, is a Washing Machine.

We use washing machines almost daily but wouldn’t get the idea that it is an embedded system consisting of a Processor (and other hardware as well) and software.

Embedded Systems can not only be stand-alone devices like Washing Machines but also be a part of a much larger system. An example for this is a Car. A modern day Car has several individual embedded systems that perform their specific tasks with the aim of making a smooth and safe. Some of the embedded systems in a Car are Anti-lock Braking System (ABS), Temperature Monitoring System, Automatic Climate Control, Tyre Pressure Monitoring System, Engine Oil Level Monitor, etc.

PROGRAMMING IN EMBEDDED SYSTEM:

As mentioned earlier, Embedded Systems consists of both Hardware and Software. If we consider a simple Embedded System, the main Hardware Module is the Processor. The Processor is the heart of the Embedded System and it can be anything like a Microprocessor, Microcontroller, DSP, CPLD (Complex Programmable Logic Device) and FPGA (Field Programmable Gated Array).

All these devices have one thing in common: they are programmable i.e. we can write a program (which is the software part of the Embedded System) to define how the device actually works.

Embedded Software or Program allow Hardware to monitor external events (Inputs) and control external devices (Outputs) accordingly. During this process, the program for an Embedded System may have to directly manipulate the internal architecture of the Embedded Hardware (usually the processor) such as Timers, Serial Communications Interface, Interrupt Handling, and I/O Ports etc.

From the above statement, it is clear that the Software part of an Embedded System is equally important to the Hardware part. There is no point in having advanced Hardware Components with poorly written programs (Software).

There are many programming languages that are used for Embedded Systems like Assembly (low-level Programming Language), C, C++, JAVA (high-level programming languages), Visual Basic, JAVA Script (Application level Programming Languages), etc.

In the process of making a better embedded system, the programming of the system plays a vital role and hence, the selection of the Programming Language is very important.

### INTRODUCTION TO EMBEDDED C PROGRAMMING LANGUAGE

Before going in to the details of Embedded C Programming Language and basics of Embedded C Program, we will first talk about the C Programming Language.

The C Programming Language, developed by Dennis Ritchie in the late 60’s and early 70’s, is the most popular and widely used programming language. The C Programming Language provided low level memory access using an uncomplicated compiler (a software that converts programs to machine code) and achieved efficient mapping to machine instructions.

The C Programming Language became so popular that it is used in a wide range of applications ranging from Embedded Systems to Super Computers.

Embedded C Programming Language, which is widely used in the development of Embedded Systems, is an extension of C Program Language. The Embedded C Programming Language uses the same syntax and semantics of the C Programming Language like main function, declaration of datatypes, defining variables, loops, functions, statements, etc.

The extension in Embedded C from standard C Programming Language include I/O Hardware Addressing, fixed point arithmetic operations, accessing address spaces, etc.

### DIFFERENCE BETWEEN C AND EMBEDDED C

There is actually not much difference between C and Embedded C apart from few extensions and the operating environment. Both C and Embedded C are ISO Standards that have almost same syntax, datatypes, functions, etc.

Embedded C is basically an extension to the Standard C Programming Language with additional features like Addressing I/O, multiple memory addressing and fixed-point arithmetic, etc.

C Programming Language is generally used for developing desktop applications whereas Embedded C is used in the development of Microcontroller based applications.

# EMBEDDED SYSTEM AND ITS REAL TIME APPLICATIONS

The World is filled with Embedded Systems. The development of Microcontroller has paved path for several Embedded System application and they play a significant role (and will continue to play in the future as well) in our modern day life in one way or the other.

Starting from consumer electronics like Digital Cameras, DVD Players to high end and advanced systems like Flight Controllers and Missile Guidance Systems, embedded systems are omnipresent and became an important part of our life.

The way we live our life has been significantly improved with the utilization of Embedded Systems and they will continue to be an integral part of our lives even tomorrow.

Another important concept we are hearing these days is Real – Time Systems. In a real time system, Real Time Computing takes place, where a computer (an Embedded System) must generate response to events within certain time limits.

Before going in to the details of Real Time Applications of Embedded Systems, we will first see what an Embedded System is, what is a real time system and what is real time operating system.

### Some Other Real Time Applications of Embedded Systems

- Latest Smart TVs

- GPS Navigation Systems

- Almost all Modern Day Smart Phones

- Missile Guidance Systems

- Space Exploration (Rovers)

- Automobiles (ABS, Airbags)

- Industries (Assembly Robots)

- Road Safety Systems (Traffic Monitoring and Collision Alert Systems)

- and many other.

PIC C COMPILER:

CCS provides a method to attempt to make sure you can compile code written in older versions of CCS with minimal difficulty by altering the methodology to best match the desired version. Currently, there are 4 levels of compatibility provided: CCS V2.XXX, CCS V3.XXX, CCS V4.XXX and ANSI.

Notice: this only affects the compiler methodology, it does not change any drivers, libraries and include files that may have been available in previous versions.

#device CCS2

• ADC default size is set to the resolution of the device (#device ADC=10, #device

ADC=12, etc)

• boolean = int8 is compiled as: boolean = (int8 != 0)

• Overload directive is required if you want to overload functions

• Pointer size was set to only access first bank (PCM *=8, PCB *=5)

• var16 = NegConst8 is compiled as: var16 = NegConst8 & 0xFF (no sign extension)

• Compiler will NOT automatically set certain #fuses based upon certain code conditions.

• rom qualifier is called _rom

#device CCS3

• ADC default is 8 bits (#device ADC=8)

• boolean = int8 is compiled as: boolean = (int8 & 1)

• Overload directive is required if you want to overload functions

• Pointer size was set to only access first bank (PCM *=8, PCB *=5)

• var16 = NegConst8 is compiled as: var16 = NegConst8 & 0xFF (no sign extension)

• Compiler will NOT automatically set certain #fuses based upon certain code conditions.

• rom qualifier is called _rom

#device CCS4

• ADC default is 8 bits (#device ADC=8)

• boolean = int8 is compiled as: boolean = (int8 & 1)

• You can overload functions without the overload directive

• If the device has more than one bank of RAM, the default pointer size is now 16

(#device *=16)

• var16 = NegConst8 is will perform the proper sign extension

• Automatic #fuses configuration (see next section)

#device ANSI

• Same as CCS4, but if there are any discrepancies are found that differ with the ANSI

standard then the change will be made to ANSI

• Data is signed by default

• const qualifier is read-only RAM, not placed into program memory (use rom qualifier to

place into program memory)

• Compilation is case sensitive by default

• Constant strings can be passed to functions (#device PASS_STRINGS_IN_RAM)

This C compiler, is fully optimised for use with PIC microcontrollers. Built in functions make coding the software very easy. Based on original K&R, the integrated C development environment gives developers a fast method to produce efficient code from an easily Maintainable high level language.

CAPABILITIES

· Arrays up to 5 subscripts

· Structures and Unions may be nested.

· Custom bit fields (1-8 bits) within structures.

· ENUMurated types,

· CONSTant variables, arrays and strings.

· Full function parameter support (any number).

· Some support for C++ reference parameters.

STANDARD C FUNCTIONS:

· IF, ELSE, WHILE, DO, SWITCH, CASE, FOR,

RETURN, GOTO, BREAK, CONTINUE

· !, ~, ++, --, *, /, %, +, -, <<, >>, <. <=, >, >=,

==, !=, &, ^, |, &&, ||, ?:, =, +=, -=, *=, /=, %=,

>>=, <<=, &=, ^=, |=

· TYPEDEF, STATIC, AUTO, CONST, ENUM,

STRUCT, UNION

FEATURES:

- Built in Libraries for RS232 serial I/O library, I/O, I2C, discrete I/Oand precision delays.

- Integrates with MPLAB and other simulators/emulators for source level debugging.

- Standard Hex file and debug files ensure compatibility with all programmers.

- Formatted Printf allows easy formatting and display in Hex or decimal.

- Efficient function implementation allows call trees deeper than the hardware stack.

- Access to hardware from easy to use C functions, Timers, A/D, E2, SSP, PSP, I2C & more.

- 1,8, and 16 bit types.

- Assembly code may be inserted anywhere in source and may reference C variables.

- Automatic linking handles multiple code pages.

- Inline procedures supported; Linker automatically determines optimum architecture or it can be manually specified.

- Compiler directives determine if tri-state registers are refreshed on every I/O

- Constants (including strings and arrays) are saved in program memory.

- Standard one bit type (Short Int) permits the compiler to generate efficient Bit oriented code.

- #BIT and #BYTE allow C variables to be placed at absolute addresses to map register to C variables.

- Reference parameters may be used to improve code readability and inline procedure efficiency.

- Both an Integrated editor/compiler and a cmd line compiler.

- Special windows show the RAM memory map, C/Assembly listing and the calling tree.

- Interrupt procedures supported on PCM. The compiler generates all startup and cleanup code as well as identifying the correct interrupt procedure to be called.

- Updates via modem for 30 days included.

DESCRIPTION

This integrated C development environment gives developers the capability to quickly produce very efficient code from an easily maintainable high level language. The compiler includes built in functions to access the PIC hardware such as READ_ADC to read a value from the A/D converter. Discrete I/O is handled by describing the port characteristics in a PRAGMA. Functions such as INPUT and OUTPUT_HIGH will properly maintain the tri-state registers. Variables including structures may be directly mapped to memory such as I/O ports to best represent the hardware structure in C. The microcontroller clock speed may be specified in a PRAGMA to permit built in functions to delay for a given number of microseconds or milliseconds. Serial I/O functions allow standard functions such as GETC and PRINTF to be used for RS-232 like I/O. The hardware serial transceiver is used for applicable parts when possible. For all other cases a software serial transceiver is generated by the compiler. The standard C operators and the special built in functions are optimised to produce very efficient code for the bit and I/O functions. Functions may be implemented inline or separate. Function parameters are passed in reusable registers. Inline functions with reference parameters are implemented efficiently with no memory overhead. During the linking process the program structure including the call tree is analysed. Functions that call one another frequently are grouped together in the same page. Calls across pages are handled automatically by the tool transparent to the user. Functions may be implemented inline or separate. RAM is allocated efficiently by using the call tree to determine how locations can be re-used. Constant strings and tables are saved in the device ROM. The output hex and debug files are selectable and compatible with popular emulators & programmers including MPLAB for source level debugging. The Professional Package (PCW) provides both compilers in a powerful Windows environment.